# Worksheet 2

Student Name: Navjot Kaur					     UID: 25MCA20078

Branch: MCA			                              	  	     Section/Group: 25MCA-1(A)

Semester: 3rd 						               Date of Performance: 18-08-2026

Subject Name: Internet of Things				     Subject Code: 25CAH-708

# 1. Aim/Overview of the practical:

Interface a buzzer, led, pushbutton (Pull Down) and LCD with Arduino Uno. Use two led and two pushbuttons. Use pushbutton as toggle switch.

- Case 1: When press button 1 once then buzzer will turn ON and print “Button1 ON” in Row 2 and “Buzzer ON” in Row 1.

- Case 2:  When we press button 1 again then buzzer will turn OFF and print “Button1 OFF” in Row 2 and “Buzzer OFF” in Row 1.

- Case 3: When we press button 2 once then LED will turn ON and print “Button2 ON” in Row 2 and “Lights ON” in Row 1.

- Case 4:  When we release button 2 again then LED will turn OFF and print “Button2 OFF” in Row 2 and “Light OFF” in Row 2.

# 2. Apparatus (For applied/experimental sciences/materials based labs):

Hardware Requirements

- Arduino Uno R3

- Breadboard

- Push Button

- LED

- Resistor (220Ω)

- LCD Display (16*2 I2C)

- Jumper Wires

- USB Cable/Power Supply

- Buzzer

Software requirements

- Arduino IDE

- Tinkercad Circuits

3. Circuit Diagram (TinkerCad):

- Case 1: When press button 1 once then buzzer will turn ON and print “Button1 ON” in Row 2 and “Buzzer ON” in Row 1.

- Case 2:  When we press button 1 again then buzzer will turn OFF and print “Button1 OFF” in Row 2 and “Buzzer OFF” in Row 1.

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

LiquidCrystal_I2C lcd(0x27, 16, 2);

int b1 = 2;

int buzzer = 8;

int buzzerState = 0;

void setup() {

pinMode(b1, INPUT);

pinMode(buzzer, OUTPUT);

lcd.init();

lcd.backlight();

lcd.print("System Ready");

delay(1000);

lcd.clear();

}

void loop() {

if (digitalRead(b1) == HIGH) {

buzzerState = !buzzerState;

if (buzzerState == 1) {

tone(buzzer, 2000);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Buzzer ON");

lcd.setCursor(0, 1);

lcd.print("Button1 ON");

}

else {

noTone(buzzer);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Buzzer OFF");

lcd.setCursor(0, 1);

lcd.print("Button1 OFF");

}

delay(300);

}

}

- Case 3: When we press button 2 once then LED will turn ON and print “Button2 ON” in Row 2 and “Lights ON” in Row 1.

- Case 4:  When we release button 2 again then LED will turn OFF and print “Button2 OFF” in Row 2 and “Light OFF” in Row 2.

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

LiquidCrystal_I2C lcd(0x27, 16, 2);

int b2 = 3;

int led1 = 9;

int led2 = 10;

int lightState = 0;

void setup() {

pinMode(b2, INPUT);

pinMode(led1, OUTPUT);

pinMode(led2, OUTPUT);

lcd.init();

lcd.backlight();

lcd.print("System Ready");

delay(1000);

lcd.clear();

}

void loop() {

if (digitalRead(b2) == HIGH) {

lightState = !lightState;

if (lightState == 1) {

digitalWrite(led1, HIGH);

digitalWrite(led2, HIGH);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Lights ON");

lcd.setCursor(0, 1);

lcd.print("Button2 ON");

}

else {

digitalWrite(led1, LOW);

digitalWrite(led2, LOW);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Light OFF");

lcd.setCursor(0, 1);

lcd.print("Button2 OFF");

}

delay(300);

}

}

5. Learning outcomes (What I have learnt):

- Learned to interface pushbuttons, LEDs, buzzer, and LCD with Arduino Uno.

- Learned to use pushbuttons as toggle switches.

- Learned to control LEDs and buzzer using Arduino.

- Learned to display ON/OFF status on the LCD.

- Learned basic digital input and output operations.